SIMATS ENGINEERING CO3- ASSESSMENT TOOL 2 SIMULATION TASK

COURSE NAME: DIGITAL SIGNAL PROCESSING	COURSE CODE: ECA09

# COURSE OUTCOME COVERED

CO 3: Evaluate finite word-length effects and multirate signal processing techniques, including decimation, interpolation, and polyphase decomposition. Bloom Level mapped-BL3,BL4,BL5

# Topics Covered:

Quantization- Truncation and Rounding errors - Quantization noise - Limit cycle oscillations –  Signal  scaling.Parametric  and  Non  parametric	Spectral estimation. Decimation-Interpolation-Sampling rate conversion-Implementation of sampling rate conversion-Polyohase Decomposition -5G Decimation/Interpolation: For mmWave beamforming.

Assessment Tool number	: CO3-AT2 Assessment Tool Weightage	: 35% Assessment Tool Name	: Simulation Task Duration of this assessment	: 60 Minutes

Marks	: 25 Marks

Type of Assessment Conduction	: Self Learning (Home Task)

- Each student will be assigned one simulation-based problem related to the course topics.

- Solve the assigned simulation problem independently.

- Develop and run the simulation using MATLAB/Simulink.

- Analyze and present the results with graphs and screenshots.

- Upload the simulation code and output to GitHub.

- Submit the GitHub repository link in Google Classroom (GCR).

Dr.

Code of Conduct:

I 	(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

# Signature of the student:

RUBRICS FOR CALCULATION:

| Simulation tools |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightag e (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Conceptual Understanding | 20% | Demonstrates strong understanding of underlying concepts in the simulation | Shows good understanding with minor gaps | Partial understanding; some misconceptions | Lacks understanding of key concepts |
| Model Design | 25% | Develops accurate, well-structured simulation/mode l independently | Develops correct model with minor errors | Model is incomplete or requires guidance | Unable to develop a proper model |
| Tool Usage | 20% | Uses simulation tools effectively with correct setup and execution | Uses tools with minor errors or guidance | Limited ability to use tools properly | Unable to use simulation tools |
| Analysis of Results | 20% | Provides clear, logical analysis and meaningful interpretation of results | Analysis is reasonable with minor gaps | Superficial analysis; limited interpretation | Unable to analyze or interpret results |
| Documentatio n | 15% | Presents results clearly with proper documentation and structured reporting | Documentatio n is adequate with minor issues | Poorly organized or incomplete documentation | No proper documentation or presentation |

# QUESTION :

# 23. Combined Decimation and Interpolation

# Simulate a multirate system using decimation and interpolation and analyze the resulting

# sampling rate conversion. (25) (BL4) (WK4)

# Given Data:

# ● Decimation Factor = 3

# ● Interpolation Factor = 2

# ● Input Sampling Rate = 18 kHz

# Tasks:

# 1. Design an anti-alias filter.

# 2. Perform decimation.

# 3. Implement interpolation.

# 4. Analyze the output spectrum.

# 5. Determine the final sampling rate.

# 6. Compare input and output signals.

# Solution:

PROGRAMING CODE (MATLAB)

% Combined Decimation and Interpolation

% Fs = 18 kHz, M = 3, L = 2

clc;

clear;

close all;

%% Input Signal

Fs = 18000;          % Input Sampling Rate

t = 0:1/Fs:0.01;     % 10 ms duration

% Signal containing frequencies below cutoff

x = sin(2*pi*1000*t) + 0.5*sin(2*pi*2000*t);

%% Anti-Alias Low Pass Filter

fc = 3000;                           % Cutoff Frequency

Wn = fc/(Fs/2);                      % Normalized Cutoff

N = 50;                              % Filter Order

b = fir1(N,Wn,'low');                % FIR LPF

x_filt = filter(b,1,x);

%% Decimation (M = 3)

M = 3;

x_dec = downsample(x_filt,M);

Fs_dec = Fs/M;

%% Interpolation (L = 2)

L = 2;

x_int = upsample(x_dec,L);

% Reconstruction Filter

Fs_out = Fs_dec*L;

fc2 = Fs_dec/2;

Wn2 = fc2/(Fs_out/2);

b2 = fir1(N,Wn2,'low');

x_out = filter(b2,1,x_int);

%% Final Sampling Rate

fprintf('Input Sampling Rate = %d Hz\n',Fs);

fprintf('After Decimation = %d Hz\n',Fs_dec);

fprintf('Final Output Sampling Rate = %d Hz\n',Fs_out);

%% Time Domain Plots

figure;

subplot(4,1,1);

plot(t,x);

title('Original Input Signal');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(4,1,2);

plot((0:length(x_filt)-1)/Fs,x_filt);

title('Filtered Signal');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(4,1,3);

stem((0:length(x_dec)-1)/Fs_dec,x_dec);

title('Decimated Signal (M=3)');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(4,1,4);

plot((0:length(x_out)-1)/Fs_out,x_out);

title('Interpolated Output Signal (L=2)');

xlabel('Time (s)');

ylabel('Amplitude');

%% Spectrum Analysis

figure;

subplot(2,1,1);

Nfft = 1024;

f1 = Fs*(-Nfft/2:Nfft/2-1)/Nfft;

X = fftshift(abs(fft(x,Nfft)));

plot(f1,X);

title('Input Spectrum');

xlabel('Frequency (Hz)');

ylabel('|X(f)|');

subplot(2,1,2);

f2 = Fs_out*(-Nfft/2:Nfft/2-1)/Nfft;

Y = fftshift(abs(fft(x_out,Nfft)));

plot(f2,Y);

title('Output Spectrum');

xlabel('Frequency (Hz)');

ylabel('|Y(f)|');

OUTPUT: